video of Dynacolor protocol contains four parts:

[tHeader][tSuperExtraIdx1][PES HEADER][Video Frame]

The last part video frame here can be a JPEG or MPEG4 frame depending on setting.

They are standard formats encoded from chip.

typedef struct

{

//total 4 byte//start///////////////////////////////////////////////////////

unsigned int Header1    :8; // [2] "@X" -- X:device type, DG200 is "2" @2

unsigned int Header2    :8; // [2] "@X" -- X:device type, DG200 is "2" @2

unsigned int Msec       :4; // 4 bits : 1/256 sec

unsigned int Source	:1; // 1 bit  : (0 - DVR, 1 - IP Dev)

unsigned int WidthBase	:2; // 0:720 , 1:704, 2:640

//

// if AudioSwitch is set, check Laudio/Raudio to decide which audio channel

// should be enabled with this video channel

//

unsigned int AudioSwitch:1; // 1 bit  : (0 - Disable(backward compatibile), 1 - Enable)

unsigned int Laudio     :1; // 1 bit  : (0 - Disable, 1 - Enable)

unsigned int Raudio     :1; // 1 bit  : (0 - Disable, 1 - Enable)

unsigned int Dummy      :2; // 4 bits : reserved

unsigned int QI         :4; // 4 bits : QI (0~16)

/////////////////end////////////////////////////////////////////////////////

unsigned int Size;      // [4]

time_t Time;            // [4]

//total 4 byte//start///////////////////////////////////////////////////////

unsigned int NTSC_PAL   :1;  // 1 bit : 0:NTSC/1:PAL

unsigned int ImgMode    :1;  // 1 bit : 0:Live/1:Playback

unsigned int Audio      :1;  // 1 bit : 0:Video/1:Audio

unsigned int DST        :1;  // 1 bit : DST

unsigned int Covert     :1;  // 1 bit : Covert

unsigned int Vloss      :1;  // 1 bit : Video loss

unsigned int AlarmIn    :1;  // 1 bit : Alarm In

unsigned int Motion     :1;  // 1 bit : Motion

unsigned int ExtraEnable:1;  // 1 bit  : 0: no extra         1: have extra

unsigned int EventEnable:1;  // 1 bit  : 0: Normal status    1: Event duration

unsigned int PPS        :6;  // 6 bits : Per-channel PPS Idx (0~127 level)

unsigned int Type       :3;  // 3 bits : type (0~7) 0: none 1: I-pic, 2: P-Pic, 3: B-Pic

unsigned int Channel    :5;  // 5 bits : Channel No (0~31)

unsigned int Chksum     :8;              // [1]

/////////////////end////////////////////////////////////////////////////////

} tBasicIdx;

typedef struct

{

//total 4 byte//start///////////////////////////////////////////////////////

unsigned int   IsDST_S_W	  :1; //   0 = summer time , 1 =winter time

unsigned int   DST_Minutes	  :8; // 0~7 = 0 ~ 120 minutes

unsigned int   OverSpeed  	  :1;

unsigned int   SkipPicCnt 	  :5;

//

// use one bit to represent exception alarm

//  - can't get exactually exception no here

//

unsigned int   Exception	  :1;

unsigned int   SuperExtraSize :16;

//total 4 byte//end///////////////////////////////////////////////////////

unsigned int   Serno;			// [4]

//

// Original tEventInfo16CH was broken 'cause of the byte alignment problem

// - use the following directly

//

// ===== Start =====

//

char AlmIn[2];		// 16 channel

char Vloss[2];		// 16 channel

char Motion[2];		// 16 channel

//

// ===== End =====

//

unsigned char  IsDVRRec:2;

//

// For TimeZone setting

// - 0 means OFF (backward compatibility)

// - Delta : 30 min

// - Start from : -12:00

// - Aka.

//    1 : -12:00 (-720 min)

//    2 : -11:30 (-690 min)

//    3 : -11:00 (-660 min)

//    ....

//   25 : +00:00 (+000 min)

//    ....

//   41 : +08:00 (+480 min)

//   42 : +08:30 (+510 min)

//   43 : +09:00 (+540 min)

//    ....

//   51 : +13:00 (+780 min)

//

unsigned char  TimeZone:6;

char Chksum;					// [1]

} tExtraIdx;

typedef struct

{

//

// the type for tSuperExtraIdx is fixed to 0

//

int type;

//

// the remain_size for tSuperExtraIdx1 is fixed to : 2x3=6 bytes

//

int remain_size;

unsigned char VSMotionState[3][2];

unsigned char dummy[2];

} tSuperExtraIdx1;

typedef struct

{

tBasicIdx Basic;

tExtraIdx Extra;

}tHeader;

Video:

#define PENTAMICRO_PES_HEADER_SIZE 32

//max total size=sizeof(tHeader)+sizeof(tSuperExtraIdx1)+PENTAMICRO_PES_HEADER_SIZE+frame size

inline unsigned char DB_CalBacChksum(tBasicIdx  *header)

{

int i;

unsigned char chksum=0,*pHeader;

pHeader = (unsigned char*)header;

if( *pHeader=='@' && *(pHeader+1)=='2')

for(i=0;i < sizeof(tBasicIdx)-1;++i)

{

chksum ^= *(pHeader+i);

}

return chksum;

}

#define  PENTAMICRO_JPEG_FORMAT_PREFIX  0xD0

#define  PENTAMICRO_MPEG4_FORMAT_PREFIX  0xE0

void DVR_MakeDrvHeader(tHeader *pHdr, int serno, int size, int pic_type)

{

//

// init pHdr

//

memset(pHdr, 0, sizeof(tHeader));

//

// set basic header

//

pHdr->Basic.Header1='@';

pHdr->Basic.Header2='2';

//

//

// FIXME :

//   Set QI as fixed value here

//   - 0: default(720x240), 1: 360x240, 2: 720x480

//

pHdr->Basic.QI = (GetVideoResolution()==0)?2:1;

//

// pic_type

// - get from raw data

//

pHdr->Basic.Type = pic_type;//frame type 1=I, 2=P,3=B

pHdr->Basic.Source = 1;//for IP dev

pHdr->Basic.Size = size;//drv header + frame

//

// Time info

//

pHdr->Basic.Time = time(NULL);

//

// NTSC_PAL

// - 7 bit : 0:NTSC/1:PAL

//

pHdr->Basic.NTSC_PAL = GetVideoType();//0=NTSC,1=PAL

//

// Audio

// - 5 bit : 0:Video/1:Audio

//

pHdr->Basic.Audio = 0;//audio=1. video=0

pHdr->Basic.ExtraEnable = 1;

//

// set extra header

//

pHdr->Extra.Serno = serno;

//

// TODO : event status

//

if(Faraday_MotionValue())

pHdr->Extra.Motion[0]|=0x01;

if(ReadAlarmValue())

pHdr->Extra.AlmIn[0]|=0x01;

pHdr->Extra.SuperExtraSize=sizeof(tSuperExtraIdx1);

//

//

pHdr->Basic.Chksum = DB_CalBacChksum(&pHdr->Basic);

pHdr->Extra.Chksum = DB_CalExtChksum(&pHdr->Extra);

}  // end DVR_MakeDrvHeader()

//stream_id ane dts can be set to 0.

//size is the frame size.

//pic_type=pHdr->Basic.Type

void DVR_MakePesPacketHeader(unsigned char *pes, int pic_type,

int size, long long dts, int stream_id, int format_prefix)

{

//

// the size should contain this 32-byte PES header.

//

size += PENTAMICRO_PES_HEADER_SIZE;

// pes packet header

// start code

pes[0] = 0x00;

pes[1] = 0x00;

pes[2] = 0x01;

// format prefix(JPEG:0xd0, MPEG4:0xe0) and stream id

pes[3] = (format_prefix | (stream_id&0x0F));

// pes packet length

pes[4] = ((size - 6) >> 8) & 0xff;        // packet_length (header 32 bytes)

pes[5] = (size - 6) & 0xff;

// flags

pes[6] = 0x87;

pes[7] = (dts>0) ? 0xc1:0x01; // valid DTS or not

// pes header data length

pes[8] = 0x17;

// pts

// pts value is different from dts value. It is complicated to calculate.

// For convenience, dts value is used in this sample code.

pes[9]  = (unsigned char)(0x31 | (((dts>>30) & 0x3) << 1));

pes[10] = (unsigned char)(0x00 | (((dts>>22) & 0xff) << 0));

pes[11] = (unsigned char)(0x01 | (((dts>>15) & 0x7f) << 1));

pes[12] = (unsigned char)(0x00 | (((dts>>7)  & 0xff) << 0));

pes[13] = (unsigned char)(0x01 | (((dts>>0)  & 0x7f) << 1));

// dts

pes[14] = (unsigned char)(0x11 | (((dts>>30) & 0x3) << 1));

pes[15] = (unsigned char)(0x00 | (((dts>>22) & 0xff) << 0));

pes[16] = (unsigned char)(0x01 | (((dts>>15) & 0x7f) << 1));

pes[17] = (unsigned char)(0x00 | (((dts>>7)  & 0xff) << 0));

pes[18] = (unsigned char)(0x01 | (((dts>>0)  & 0x7f) << 1));

// flags

pes[19] = 0x0f;

// pes extension field length

pes[20] = 0x8b;

// previous pes packet length (not specified in this sample code)

pes[21] = 0;

pes[22] = 0;

pes[23] = 0;

// current pes packet length

pes[24] = 0x80 | (size>>15);

pes[25] = (((size>>11)&0xf)<<4) | 0x08 | ((size>>8)&0x7);

pes[26] = size & 0xff;

//

// additional flags (... picture coding type)

// this flag is not used in AT204x, it is just for application usage.

//

pes[27] = (unsigned char)pic_type;

// reserved

pes[28] = 0xff;

pes[29] = 0xff;

pes[30] = 0xff;

pes[31] = 0xff;

}  // end DVR_MakePesPacketHeader()

audio of Dynacolor protocol contains two parts:

[tHeader][Audio Frame]

Audio:

//audio is ms adpcm. Each package size is 32(tHeader) +2048(ms adpcm).

tHeader *Header= frame;

Header->Basic.Header1='@';

Header->Basic.Header2='2';

Header->Basic.Size=2048; //

Header->Basic.Time=time(NULL);

Header->Basic.Audio=1;  //    5 bit : 0:Video/1:Audio^M

Header->Basic.ExtraEnable=1;

Header->Extra.Serno=audioserno++;

for(i=0; i<sizeof(tBasicIdx) - 1; ++i)

chksum ^= *(frame+i);

Header->Basic.Chksum=chksum;