7/27/12

###### Revision 24

NEXIO™

Video Server File System

## Table of Contents

Table of Contents	2

Revision History:	2

1.0 Introduction	3

1.1 Overview	3

1.2 Streaming Formats	4

2.0 Nexio File Access Mechanisms	4

2.1 Obtaining Directory information Using LLM Protocol	4

2.2 Reading and Writing Using LLM Protocol	5

2.3 Application Specific Streaming Handlers	7

3.0 The Nexio Native Stream Format (LXF)	9

3.1 LXF Packet Overview	10

3.2 Packet Header	10

3.3 Video Packet	11

3.4 VBI Format	12

3.5 Discontinuous ltc/vitc/afd vbi packet	13

3.6 Audio Packet	13

3.7 Segment Packet	14

3.8 Disk_Segment and ID Structure	14

4.0 Sample Application and Code	15

5.0 Annexes	16

5.1 New LXF Format	16

5.2 Old LXF Format	20

5.3 Extended Fields	23

## Revision History:

| Date | Version | Name | Change |
| --- | --- | --- | --- |
| 11/26/02 | 6 | T. Roth | Added update for xFields, added DISK_SEGMENT *Transcode41.zip needs update! |
| 6/24/03 | 7 | T. Howard | Transcode.zip updated LLMLib.zip updated |
| 2/18/04 | 8 | T. Howard | Specify that video picturesize must be a multiple of 8 bytes. Fill with nulls as necessary. |
| 8/12/04 | 9 | T. Howard | VR references replaced by Nexio references Expand on the Extended Fields structure New Aspect Ratio added to VR structure |
| 5/19/07 | 10p | T. Roth | Updated to include: - new byte streaming DTA interface - new lxf format Reorganized document New embedded components needed - xcode.zip (transcode) - llmlib.zip - streaminghandles2 |
| 1/28/08 | 11p | T.Roth | Added User segments table, VBI format |
| 2/28/08 | 12p | T.Roth | Updated video formats |
| 7/11/08 | 13p | T.Roth | Removed “additional directory commands.txt” now in updated SDK 3.3 Added new DTA modes |
| 8/14/08 | 14p | T. Roth | Fixed video format – added aspect ratio bit to sec 5.1 Disk Segment |
| 8/15/08 | 15p | C. Cruz | Added description for discontinuous ltc/vitc/afd packet |
| 10/1/08 | 16p | T.Roth | Added DVC Pro 100 video format |
| 11/3/08 | 17p | T. Roth | Fixes in video format info. Fixed packet durations units (sec 3.2), Audio packet sizes (sec 3.6) |
| 12/10/08 | 18p | T.Roth | Updated Extended field metadata info Added 20->24 bit audio sample unpacking example |
| 1/14/09 | 19p | T.Roth | Revised Video Format Table (5.1.1) to clarify DVCPro HD detection Clarified Sec 5.1 “duration” field for progressive systems |
| 2/5/09 | 20p | T.Roth | Added to SEC 3.4 VBI format |
| 2/27/09 | 21p | T.Roth | Fixed Extended Fields sec 5.3, added example |
| 3/24/09 | 22p | T.Roth | Fixed SOM and TC type info in NEW LXF data segment |
| 9/28/10 | 23p | H.Wang | Revised extended field 23 definition and added extended field 24 |
| 7/27/12 | 24p | H. Wang | Added VBI trunk size should keep constant for a given file |

## 1.0 Introduction

The aim of this document is to provide a deeper understanding of access to the Nexio Server system’s than can be found in our standard protocol SDK documentation. The interface described herein to the server’s primary executable, the LLM.EXE, is a more intimate integration and has the capability of causing harm to a video server system if not attempted with caution and care. No initial integration should be attempted on an on-air system without thorough testing of all phases of the server’s capabilities.

This document will include detail and sample code as to how the LLM works and how a third party could integrate with it. There are several methods to go about this. This document is broken up into the different ways of examining our file system, reading from it and writing to it, how to move different types of files to it, and interfacing with our protocol.

### 1.1 Overview

Generally speaking video servers can take several approaches to storage at the following 2 levels:

- The file system: can be either a standard one (e.g. Microsoft NTFS, possibly with disk striping) or a proprietary one.

- The video file format: can be either a standard one (e.g. AVI, OpenDML-AVI, QT, AAF, MXF), or a proprietary one.

Broadcast or on-air video servers such as Nexio require deterministic performance (bandwidth and access latency) to every media file they store.  They also need to intrinsically support steam-oriented workflows such as playback during live capture.  These requirements lead to certain tradeoffs and features that vary significantly from standard file system design.

Nexio file system features:

- Guaranteed media access bandwidth – reserved bandwidth for real-time ingest and playback of compressed media files (typically 15-150 Mbps).

- Guaranteed media access latency – sub 500ms access to ANY frame of ANY file

- Intrinsic data protection – Software Raid 3 and ECC as well as intrinsic mirroring.

- Ability to create “looping” media – For example, a 1 hour piece of media that overwrites itself.

The Nexio server systems use a proprietary file system structure, mainly due to the unavailability of a cluster OS with disk striping and software error correction at the time when development started for its predecessor, the VR Server system. Given the high cost of supporting fully buffered generic file I/O for such a proprietary file system, the Nexio has also chosen a proprietary video file format that only requires unbuffered block I/O. In addition to being proprietary, this video file format is tightly coupled with the file system itself so the latter could be better described as a Video File System.

Nexio file system tradeoffs:

- Fixed, large allocation unit size (typically > 4MB) – inefficient for small file operations.

- Unknown file size during record – “growing” files are difficult for file transfer operations.

- Flat file system – 8 byte file handles + extended names

Most notably, some of the frame index information of each video file is not part of that file’s address space, but rather belongs to the file system’s File Allocation Table (FAT), which is always resident in memory. The LLM contains a mechanism that broadcasts changes to the FAT to other LLM’s over Ethernet. Similarly, some of each file’s metadata is stored directly in the file system’s directory area.

Due to the above format considerations, and also due to the need to maintain consistency among the cached file system data among the various Nexio servers that compose a cluster, direct low-level access of external applications to the Nexio’s file system is not possible.

The question of allowing video file exchange between different Nexio clusters or between a Nexio cluster and an external application was solved by implementing 2 commands: Generic File Write and Generic File Read. These commands can be used to move a video file between the Nexio and an external byte stream. This byte stream can be a TCP socket, a standard OS file, or a dedicated I/O device supporting sequential byte access.

### 1.2 Streaming Formats

Streaming can be accomplished in several formats:

- The Nexio native stream format, also known as the LXF format. This is the most efficient format and can fully represent any Nexio file. It is as close as it can get to the actual internal file format stored on a Nexio storage system (see mode 6 below).

- Standard formats such as WAV, AVI, QT, MXF. There is not always a 1-to-1 mapping between each format and the Nexio internal file format and costly conversions may be required. For example, when the Nexio file uses non-interleaved 20-bits/sample packed audio, which doesn’t have a direct equivalent in the external format.

- Application-specific stream formats. Any such formats can be supported by writing a Streaming Handler, which is a component that the Nexio uses, when required, to stream to that particular format. Streaming can occur to/from a TCP socket, an OS file, or any I/O device designated by the context information passed in the Generic File Read/Write Commands. The advantage of using such an application-specific handler as opposed to transferring the Nexio native stream format to an external .EXE application via socket for subsequent parsing/translation is that it eliminates one level of parsing and several levels of copying associated with the socket transfer. The Streaming Handler has direct access to the Nexio’s data buffers and transfers the data in frame multiples. However, efficiency issues requiring a custom Streaming Handler will typically not occur for transfers below 200 Mbits/second aggregate bandwidth.

## 2.0 Nexio File Access Mechanisms

### 2.1 Obtaining Directory information Using LLM Protocol

In addition to the 8-byte ID code (called just "ID" in  what follows), an extended name field has been added (called "Extended Name" in what follows). The maximum size in bytes of the Extended Name is specified at volume initialization time (16, 32, 64). An option will be made available to convert existing volumes so that they also support Extended Names. The Extended Name can contain any byte values, including embedded 0's because its size is always determined by byte count. It is treated as opaque data by the LLM. Spot search by Extended Name requries exact match. The controller should therefore adhere to a consistent strategy of either trimming trailing spaces or padding with spaces up to the maximum length. The later is preferred for future compatibility with Louth 32.

The standard mechanism for getting a directory is based on a modified list first/ list next approach.

The basic approach is:

BuildInitalIndex{								// Build initial list

BOOL bFound;

CLeitchSocket m_LLM;

CVrId id;										// 8 byte handle

CVrxId xid;										// 16, 32 or 64 byte extended handle

m_LLM.StartSocket(LLM_TIMEOUT, m_LLMHost); 	// Open LLM Communication Socket

// Build initial list

// List First List (C0 4C))

m_LLM.ListFirstListId(	0x21					// Main ID Handle / Extended ID List (0x21)

id, 					// 8 byte handle

xid, 					// 16, 32 or 64 byte extended handle

bFound);				// success

while (bFound)

{

// the accessing application should keep a local table / database of media

// other calls such as: 	query ID size in bytes (CA C9)

//							get id metadata (C8 4A)

//							get extended field (C9 C3)

//							get special ID attributes (C8 84)

AddClip(id, xid, <data>, TRUE);			// build local table

m_LLM.ListNextId(id, bFound);				// List Next A0 15

}

// set change notification flag

// m_LLM.ChangedNotify((IDS_IN_ADDED_LIST |	// new ID added

IDS_IN_DELETED_LIST |	// ID deleted

RECORDING_STARTED |	// ID entered record

RECORDING_STOPPED));	// ID finished record

}

When change notifications are received a similar process can be used to update the application directory.

The related commands are found in the Nexio Server SDK Vol1 – Protocol document (see Building a Database of Media and Managing Media Assets sections).

### 2.2 Reading and Writing Using LLM Protocol

Third party applications can make use of LLM protocol commands to send to the Nexio video/audio data within the specific formats listed below. The third party application need only construct a socket connection with the LLM and send the corresponding command to the LLM.

Harris has source code available (see attached Llmlib.zip file) to help make that connection to the LLM. It is in an LLM Library file that some of our developers use to access the functions of the LLM. Instead of writing their own socket classes, third party developers can include the llmlib.h file in their compiled code and llmlib.lib in their linked libraries. It is a statically linked library that can use the same type of build (debug/release) as the project being created.

NEEDS UPDATE

Once connected to the LLM, the major functions required to read from and write to our Video File System are the Generic File Read command, CF A0 and the Generic File Write command, CF A1.

#### 2.2.1 SUPPORTED DATA ACCESS MODES

The following table lists supported data access modes via existing stream handlers.

| Mode | Description | OS File | DTA Socket | Handler params | Read Only | Write Only |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | Raw user data (not interpreted by LLM) |  |  |  |  |  |
| 1 | ALPHA data. The alpha blocks should not exceed the allocation unit size of the disk (available through the C0 B2 command), or otherwise the LLM will end the transfer with a TRANSACTION_INVALID_FORMAT status |  |  |  |  |  |
| 4 | MPEG ES (block structured interleaved video and audio) |  | X |  | X |  |
| 5 | MPEG ES (block structured interleaved video and audio) | X |  |  | X |  |
| 6 | LXF (block structured interleaved video and audio) |  | X |  |  |  |
| 7 | LXF (block structured interleaved video and audio) | X |  |  |  |  |
| 8 | WAV format |  | X |  |  |  |
| 9 | WAV format | X |  |  |  |  |
| 10 | AVI format (including OpenDML spec for >2GB files) |  | X | X |  |  |
| 11 | AVI format (including OpenDML spec for >2GB files) | X |  | X |  |  |
| 12 | DV/DIF |  | X |  |  |  |
| 13 | DV/DIF | X |  |  |  |  |
| 14 | MXF |  | X |  |  |  |
| 15 | MXF | X |  |  |  |  |
| 16-31 | User segments – 16 independently accessed data spaces (see annex 5.32 table) |  |  |  |  |  |
| 33 | TGA files | X |  |  |  |  |
| 34 | LXF – video only |  | X |  | X |  |
| 35 | LXF – video only | X |  |  | X |  |
| 36 | LXF – audio only |  | X |  | X |  |
| 37 | LXF – audio only | X |  |  | X |  |
| 44 | LXF – video headers only |  | X |  | X |  |
| 45 | LXF – video headers only | X |  |  | X |  |
| 46 | MXF for XDCAM |  | X |  |  |  |
| 47 | MXF for XDCAM | X |  |  |  |  |
| 48 | MOV Quicktime |  | X | X |  |  |
| 49 | MOV Quicktime | X |  | X |  |  |
| 50 | AIFF |  | X | X |  |  |
| 51 | AIFF | X |  | X |  |  |
| 52 | MPEG2 PS and SPTS – Import |  | X |  |  | X |
| 53 | MPEG2 PS and SPTS – Import | X |  |  |  | X |
| 54 | GXF |  | X |  |  |  |
| 55 | GXF | X |  |  |  |  |

#### 2.2.2 GENERIC FILE READ COMMAND

To read from our Video File System, the Generic File Read command, CF A0 is used. What follows is a description of this command:

Available read modes:

CF A0 10 <DTA type (1)> <DTA specification (6)> <mode (1)>  <file name (8)>

OR

CF A0 18 <DTA type (1)> <DTA specification (6)> <mode (1)> <file name (8)> <field offset (4)> <field count (4)>

OR

CF A0 1C <DTA type (1)> <DTA specification (6)> <mode (1)> <file name (8)> <field offset (4)> <field count (4)> <in TC (4)>

OR

CF A0 <0x1C + p (1)> <DTA type (1)> <DTA specification (6)> <mode (1)> <file name (8)> <field offset (4)> <field count (4)> <in TC (4)> <handler specific params (p)>

Available DTA types:

- TCP Server DTA:

<DTA type>=00 , <DTA specification>=<client address (4)> <client port (2)>

Prior to issuing the command the client needs to connect a SOCK_STREAM to port 560 of the machine running the LLM, then use getsockname() to retrieve the <client address>, <client port> that need to be used in this command. If it is known that there can't be more than one such data connections active at any given time (no parallel file transfers) then a value of 0, 0 could also be used for <client address>, <client port> which means the LLM can pick any connection.

- TCP Client DTA:

<DTA type>=01 , <DTA specification>=<server address (4)> <server port (2)>

Prior to issuing the command the client needs to create a server side SOCK_STREAM (socket(), bind(), listen()), pass its coordinates <server address>, <server port> as the DTA specification, and prepare to accept a connection (accept()). The LLM data agent will connect as a client to the specified socket. If the connection cannot be established the transaction will end in TRANSACTION_CONNECTION_FAILED. Other than that, the connection will be established even if the transaction ends in TRANSACTION_NO_FILE or other errors, and then it will be closed.

- TCP socket handle DTA:

<DTA type>=02 , <DTA specification>=<socket handle (4)> <process Id (2)>

Prior to issuing the command the client needs to create the data connection by using either connect() or accept(), then pass the data socket's handle and its own process Id to the LLM. Once the command has been issued, the controller shall not access the data socket's handle any more, since it will be auto-closed by the LLM. Both <socket handle> and <process Id> are represented MSB first.

Handler Specific Parameters (QT, AVI,AIFF):

<handler specific params (p)> = <byte offset (4)> <byte count (4)>

The data returned by the read command contains: bytesToRead (__int64) + newFileOffset (__int64) + data.

- bytesToRead usually equals byteCount, unless the end of file was reached or passed (in this case bytesToRead < byteCount);

- newFileOffset is usually byteOffset+byteCount, unless the end of file was reached or passed (in this case newFileOffset = fileSize);

- special case: byteOffset=_I64_MAX and byteCount=0 -> returns bytesToRead=0 and newFileOffset=fileSize (used to get the file size);

#### 2.2.4 GENERIC FILE WRITE COMMAND

To write to our Video File System, the Generic File Write command, CF A1 is used. What follows is a description of this command:

Available write modes:

CF A1 12 <DTA type(1)><DTA specification(6)><mode(1)><file name(8)><attribute bits(2)>

OR

CF A1 16 <DTA type (1)> <DTA specification (6)> <mode (1)><file name (8)><attribute bits (2)>

<in TC (4)>

OR

CF A1 <0x16 + p (1)> <DTA type(1)> <DTA specification (6)> <mode (1)> <file name (8)> <attribute bits (2)> <in TC (4)> <handler specific params (p)>

The client will close the connection after all the data is sent. The LLM will close the connection only if an existing file was write protected, the disk is full, or an unrecoverable error occurred.

The file attributes will be set according to <attribute bits>.

For data type=1,3,5,6 <in TC> is the time code location where the written material needs to start. If absent, it is assumed to be 00:00:00:00 for types 1 and 3 or taken from the ID for types 5 and 6.

When the source is an NT file (data types 7, 9, 11, and 15), the full ASCII name of the desired file needs to be sent to the LLM DTA socket instead of the actual data, and then the connection must be closed. Actual data transfer from the file will only commence after the DTA connection is closed.

All the numeric fields are represented MSB first.

More information is available regarding both the Generic Read File command, CF A0, and the Generic Write File command, CF A1, in the attached file.

### 2.3 Application Specific Streaming Handlers

A method for transferring video/audio data from/to the LLM in other than the native stream format or the few supported standard formats is implementing custom streaming handlers. A Streaming Handler is tightly integrated at a low level with the LLM and will be invoked when a Generic File Read/Write command is processed for that specific format. For the write-to-server direction the handler is a class that implements the IWriteHandler interface. This class in turn invokes methods of the IWriteProcess interface that is made available by the LLM.

The definition of IwriteProcess and IwriteHandler are below:

class IwriteProcess

{

public:

virtual int Receive(void * buf, int length)=0;

virtual void Done(TRANSACTION_STATUS transactionStatus)=0;

virtual BOOL PutVideoPacket(

int start,

int duration,

ULONG format,

ULONG N,				 // GOP size

ULONG M,				 // reference picture period

ULONG bitRate,

ULONG pictureType, //0=I(closed),1=I(open), 2=P, 3=B

ULONG pictureSize,

ULONG vbiFormat,

ULONG vbiPictureSize,

ULONG metaFormat,

ULONG metaPictureSize,

BOOL contiguous)=0;

virtual BOOL PutAudioPacket(

__int64 start,

__int64 duration,

__int64 firstSample,	//first active field

__int64 samples,		//total samples,

//samples>=firstSample+duration

ULONG sampleSize,		// in bits

ULONG samplePrecision, // in bits

ULONG trackMask)=0;

virtual void PutFileInfo(

int startOffset,

int duration,

DISK_SEGMENT * ps,

ULONG infoMask

void * pxFields,

int xFieldSize,

int xField )=0;

virtual void PutFileInfo(

int startOffset,		// in fields

int duration,			// in fields

ULONG startTC,		// BCD_TC

ULONG format,

ULONG N,				// GOP size

ULONG M,				// reference picture period

ULONG bitRate,

BOOL vbiPresent,

int	videoGain,

int	videoSetup,

int	chromaGain,

int	hue,

ULONG audioTracks,

const SYSTEMTIME * pRecordDate,

const SYSTEMTIME * pKillDate,

const char * description,

const char * agency,

const char * type,

ULONG infoMask,

void * pxFields=NULL,

int xFieldsSize=0,

int xField=0)=0;

};

class IWriteHandler

{

public:

virtual void Run(IWriteProcess * pWriteProcess, BOOL _fileMode)=0;

virtual BOOL TransferAudioData(

UCHAR * address,

ULONG offsetSamples, //offset & size are in samples

ULONG sizeSamples,

int tracks,

ULONG trackStride)=0;

virtual BOOL TransferVideoVBIData(UCHAR * address, ULONG size)=0;

virtual BOOL TransferVideoData(UCHAR * address, ULONG size)=0;

};

Depending on the data format needed to input to the raid, the LLM chooses the correct IwriteHandler and calls its member function:

IWriteHandler :: Run(IWriteProcess * pWriteProcess, BOOL _fileMode)

The LLM will give IwriteHandler the pointer of object IwriteProcess in pWriteProcess. The parameter _fileMode tells if the input is from NT files or over a DTA socket.

The implementation of IWriteHandler :: Run(…) should call pWriteProcess ->PutFileInfo(…) to tell the LLM the general video/audio data information, then call the pWriteProcess->PutVideoPacket(…) and pWriteProcess-> PutAudioPacket(…) to tell the LLM each video and audio packet information. Usually the packet is one frame of data.

The functions pWriteProcess->PutVideoPacket(…) and pWriteProcess-> PutAudioPacket(…) will put the necessary information for each video/audio frame to the raid and then call back IWriteHandler -> TransferVideoData(…) to inquire about the actual video data transferring to the buffer. This is the same case for audio data transferring except it needs to call IwriteHandler-> TransferAudioData(…) to inquire about the audio data into a specific buffer.

Please note in this model the data transfer flow is controlled, and the buffers hosted, by the LLM process. For any specific video/audio format, developers just need to implement a corresponding IWriteHandler  as an in-process component. If serious reasons require the streaming handler to be a separate (but local) process, direct access of the handler to the LLM’s buffers is still possible via a shared memory mechanism that is transparent to the streaming handler.

For reference, the source code of several streaming handlers is included:

dtahinteraces.h – definition of the IWriteHandler and IWriteProcess interfaces

dtahpacket.h, .cpp – streaming handler for the VR native format (Yes, even the native format is implemented internally by a handler)

dtahavi.h, .cpp – streaming handler for the standard AVI format

dtahwav.h, .cpp – streaming handler for the standard WAV format

NEEDS UPDATE

## 3.0 The Nexio Native Stream Format (LXF)

This format contains a sequence of packets representing video, audio and segment data. Each packet is made up of a header followed by data (payload). The packet headers are of fixed size (implemented by a union over all cases), but the data that follows after each header is of variable size. The data size is as follows:

- Video (type=0): header.info.video.vb.pictureSize + header.info.video.active.pictureSize

  - VBI data comes first if present, then elementary essence follows

  - The size of the video data must be a multiple of 8, padded with nulls if necessary

- Audio (type=1): header.info.audio.trackSize * <# of 1's in header.info.audio.trackMask>

- Segment (type=2): header.info.segment.dataSize + header.info.segment.extendedFieldsSize

  - The data consists of a DISK_SEGMENT structure followed by the extended fields area: the last header.info.segment.extendedFieldsSize bytes. The extended fields area is a concatenation of all the extended fields including the empty ones. Each extended field is represented as a 1-byte byte count (0-255) followed by the actual data bytes.

  - The header.info.segment.extendedFieldsSize is only relevant if header.info.segment.format=1. This new packet format was defined to support the new fields of metadata associated with each clip. The LLM still supports the old format (header.info.segment.format=0) at the input, but it always exports with header.info.segment.format=1. As a result, even if extended fields are not enabled on a system, it is still recommended that integrators set the format to 1.

Right now in the LXF format, only the first packet is segment data. Each video packet in Nexio MPEG and DV systems contain one frame of data. Audio packets contain 5 frames (8008 samples * # of channels) for NTSC and 1 frame (1920 samples * # of channels) for PAL.

### 3.1 LXF Packet Overview

Based on the LXF stream format, you can expect the packets to be organized as follows, where S=segment packet, V=video packet, and A=audio packet.

NTSC:		[S1] [V1] [V2] [V3] [V4] [V5] [A1-5] [V6] [V7] [V8] [V9] [V10] [A6-10]

PAL:		[S1] [V1] [A1] [V2] [A2] [V3] [A3] [V4] [A4] [V5] [A5]

Segment:	[S1] = [{packet_header (segment)}, {DISK_SEGMENT}]

Video:		[V1] = [{packet_header (active)(vb)(metadata)}, {vbi data - if present}, {raw video ES data}]

Audio (NTSC):	[A1] = [{packet_header (audio)},

{	track0(sample1…8008), track1(sample1…8008),

track2(sample1…8008), track3(sample1…8008),

track4(sample1…8008), track5(sample1…8008),

track6(sample1…8008), track7(sample1…8008)	}]

### 3.2 Packet Header

The following functions generate and check the packet header's checksum. Data is not checksummed. See Annex 5.1 for data layout.

class PACKET_HEADER
{
public:
// Identifies this structure as an LXF header. Should contain "LEITCH" followed by zero padding.
char logo[8]; 1

// Version of this structure. The version number will be incremented with subsequent versions:
// - 0 = old version, start and duration are int's, in fields.
// - 1 = advanced version, start and duration will be __int64's, in (1/720000) sec units (e.g. 24024 = 1 frm @ 29.97fps, 28800 = 1frm @ 25 fps)

int version;2

// Size of this header in bytes.
ULONG size;3

// Specifies what type of payload is carried in the packet (i.e. audio, video, segment). This must be one of:
//
// PACKET_TYPE_VIDEO
// PACKET_TYPE_AUDIO
// PACKET_TYPE_SEGMENT

PACKET_TYPE type;4

// All LXF packets associated with a particular audio or video stream must have a unique stream identifier. For the current VR file system  //must be 0 because only one video// and one audio stream per file are supported. Multiple audio services can be supported by embedding 
// them in different AES tracks (32 available).

int stream;5

// old version:  the field number at which presentation of the LXF packet begins. For now these values increase by 2 for each subsequent 
// video frame, 10 for each audio frame.
//new version:  the time unit at which presentation of the LXF packet begins.
// int start;6

// old version: presentation duration of the current LXF packet in fields. Currently,  this field is set to 2 for video (normal interlaced), set to 10 // for audio (8008 samples at 48khz = 5 interlaced frames = 10 fields).
//
// new version: presentation duration of the current LXF packet in time units. AT 29.97 fps: video=24024, audio=120120 (nominal) 
int duration;7

}

The next section depends on if the packet type is video, audio or segment.

### 3.3 Video Packet

The video packet always contains one frame or two fields of data.

// For type == PACKET_TYPE_VIDEO
//
// The header will be immediately followed in this order by:
// 
// info.meta.pictureSize bytes of metadata.
// info.vb.pictureSize bytes of vertical blanking data
// info.active.pictureSize bytes of coded video data
//
struct
{

struct
{
// Specifies video frame format. Must be one of the 
// enum VIDEO_FORMAT constants. For transmission applications
ULONG format:4;V1

// These fields (N, M) indicate GOP structure. We have been 
// recommended to code them as 15, 3 respectively. My understanding 
// is that they are used as hints, but do not need to accurately 
// specify what is actually coded.
//
// GOP length (distance between I frames, not GOP headers)
ULONG N:7;V2 // GOP size

// Distance between anchor (reference) pictures.
ULONG M:3;V3 // reference picture period

// Average Bitrate of the coded video in Mbits/s=1000000's of bits/s
ULONG bitRate:8;V4

// MPEG picture_coding_type with added value of zero to specify
// an I-frame at a closed GOP.
ULONG pictureType:2;V5 // 0 - I (closed) , 1 - I (open), 2 - P , 3 - B

ULONG reserved:8;V6

// Amount of coded video data (in bytes) immediately following this
// LXF header.
ULONG pictureSize;V7
} active;

struct
{
ULONG format;V8 // 0

ULONG pictureSize;V9 // Amount of VBI data (in bytes) immediately preceding the coded data, use 0 for ATSC.
} vb;

struct
{
ULONG format;V10 // 0

ULONG pictureSize;V11 // Amount of metadata (in bytes) immediately preceding the VBI data, use 0 for ATSC.
} meta;

} video;

From this file format, an elementary stream can be derived simply by removing the VBI data (if present) and concatenating the consecutive video packets (minus the headers).

### 3.4 VBI / VI / VANC Format

The LXF VI format is the same for SD and HD and covers both VBI and VANC data. While SD usually contains line-oriented waveforms, so the data will represent the pixel data, however if there are ANC packets (such as AFD) then that data is carried as well. For the case of SD waveform data and ANC may not be on the same line. HD always contains ANC packets; therefore in this case the lxf VBI data represents the 8 significant bits of the anc data.

The 8 bit vertical interval data values are stored the same for both SD VBI and HD VANC.Either way the horizontal resolution of the video determines the number of pixels per line or ANC data samples.

In the case of SD VBI the 720 8 bit (luma / chroma) values per line represent pixels, in the case of SD and HD VANC the 1920 luma (1080i) or 1280 luma (720p) or 480 chroma/luma (SD) 8 bit values represent VANC packet data. Either way the data block is written and read in the same manner.

VBI data trunk size should remain constant throughout a given LXF file. So please set maximum VBI line and keep it the same for the whole file.

Note: Subsampled formats (DV100, XDcamHD35 and AVCI50) have fewer data samples per line.

VI Data Block / Picture Format

------------------ --------------------------------------------------------------------------------------------------------------------------------------

| VbiHeader                		| Y_Line[0]    	| Y_Line[1]    	| ... | Y_[lines-1]  	| Filler                      				|

|                          			|              		|              		|     |              		|                             				|

| size = VbiHeader::Size() 	| size = width 	| size = width 	| ... | size = width 	| size=(maxLines-lines)*width 	|

--------------------------------------------------------------------------------------------------------------------------------------------------------

class VbiHeader

{

public:

UCHAR maxLines;			// lines allocated

UCHAR lines;				// lines used

struct

{

UCHAR fieldLine:7;		// the 7 LSB

UCHAR field:1;			// the 1 MSB

} linePosition[1];				// source raster position of Y_Line[i], variable length

ULONG Size() 				// actual size

{

return (FIELD_OFFSET(VbiPicture,line)+maxLines*sizeof(UCHAR)+7)&~7;	// 8-bytes aligned

};

};

where:

width - the active line width of the scanning system (720, 1280, or 1920)

linePosition[k] - source raster position of Y_Line_[k] expressed as a (field, fieldLine) pair.

- interlaced scan systems of L lines per frame this translates to scan line # = ((L+1)/2)*field + fieldLine

- progressive systems scan line # = fieldLine.

Y_Line[k] - 8-bit pixel luma component.

In the case of waveform encodings (SD 480i, 576i) the 8 MSB of the 10-bit SDI samples are stored.

In the case of ancillary encodings (HD 720p, 1080i) the 8 LSB out of 10 are stored.

Exceptions for SD (480i, 576i):

------------------------------

1) If the active video compression format is DV,DVCPRO or DVCPRO50 then:

scan line # = ((L-1)/2)*field + fieldLine

Due to the fact that the VBI sampling window is considered "Bottom Field First" by Digiserver in order to make it consistent with the DV sampling window. Only affects F2 (field=1).

2) The Y_Line[]'s coming from F1 (field=0) are left-shifted by 8 pixels (bytes) with the loss of the first 8 pixels (bytes)*:

if(linePosition[k].field==0) then

{

Y_Line[k][i] = ActivePixel[i+8].Y (i=0,..,width-8-1)

Y_Line[k][i] = undefined          (i=width-8,..,width-1)

}

else

{

Y_Line[k][i] = ActivePixel[i].Y (i=0,..,width-1)

}

*Introduced by a change in Digiserver VBI buffer layout, from a 16-byte per frame header to individual, per field, 8-byte headers.

### 3.5 Discontinuous LTC/VITC/AFD VBI Packet

When recording an ID, a discontinuity packet is stored in the first available vbi line.  Since this packet is for internal use, it is stripped during playback.  The packet specifies frame offset since last occurrence of discontinuity in timecode or afd.  Conforms to type 2 ancillary data packet specified in SMPTE 291M.  DID is set to 0x0d and SDID is set to 0x0c.  Data count DC is 12 which consists of 3 4-byte values.  LTC offset is first, followed by VITC offset and AFD offset.  Last word in the packet is the checksum CS.

### 3.6 Audio Packet

The audio packet nominally contains five frames (8008 samples X number of channels) of data.

Note: 8008 is the “nominal” sample count per audio packet. However the lxf format does NOT guarantee 8008 samples per packet in general for a number of other reasons, for example if the spot was trimmed, subclipped etc.

Example: trimmed clip - this clip has negative start times for video.

Video:

PH-vr=1,sz=72,t=0,sm=0,st=-48048,dr=24024,fmt=2,N=15,M=3,br=35,ptyp=0,psz=212516,trf=2,clsd=1,tc=00:00:00:00(NDF)

PH-vr=1,sz=72,t=0,sm=0,st=-24024,dr=24024,fmt=2,N=15,M=3,br=35,ptyp=3,psz=66425,trf=0

PH-vr=1,sz=72,t=0,sm=0,st=0,dr=24024,fmt=2,N=15,M=3,br=35,ptyp=3,psz=61791,trf=1

Audio:

PH-vr=1,sz=72,t=1,sm=0,st=0,dr=72060,sSz=20,sPr=20,Msk=0000000F,tSz=12012

Nominal duration for audio packet is 120120 (5 frames) in this cases the packet has a duration of 72060, so the number of samples would be 8008*(72060/120120) = 4804 samples

What follows is the data format for the audio packet.

// For type == PACKET_TYPE_AUDIO

struct

{

int firstField; 				//first active field – OLD VERSION ONLY

int fields; 					//total fields,fields>=firstField+duration – OLD VERSION ONLY

ULONG sampleSize:6; 		//bits to store a sample

ULONG samplePrecision:6; 	//actual bits used by sample

ULONG reserved:20;

ULONG trackMask;			//bitmap indicating track present

ULONG trackSize; 			//in bytes

} audio;

Audio sample data is represented by sample size and sample precision. This was just to leave open the possibility of supporting some other formats that used to have the samples aligned on other multiples than their active size, like packing 20 bits into three bytes. This would look like:

sampleSize=24 ; samplePrecison=20

However, right now these are the only supported values:

sampleSize=samplePrecision=16

sampleSize=samplePrecision=20

sampleSize=samplePrecision=24

sampleSize=samplePrecision=32

In other words, everything is tightly packed.

The following example shows how to unpack and convert 20 bit samples to 24 bit

Two 20-bit audio samples in memory (5 bytes):   Din[5] = 01 23 45 67 89

1st Sample

Dout[i+0] = (byte)((Din[0] & 0x0F) << 4);

Dout[i + 1] = (byte)(((Din[1] & 0x0F) << 4) | ((Din[0] & 0xF0) >> 4));

Dout[i + 2] = (byte)(((Din[2] & 0x0F) << 4) | ((Din[1] & 0xF0) >> 4));

2nd Sample

Dout[j + 0] = (byte)(Din[2] & 0xF0);

Dout[j + 1] = Din[3];

Dout[j + 2] = Din[4];

### 3.7 Segment Packet

What follows is the data format for the segment packet.

// For type == PACKET_TYPE_SEGMENT
//
struct
{

ULONG format;S1   // for ATSC use SEGMENT_PACKET_FORMAT_1
ULONG dataSize;S2
// end of SEGMENT_PACKET_FORMAT_0
ULONG extendedFieldsSize;S3   // for ATSC use 0 
// end of SEGMENT_PACKET_FORMAT_1

// The data that follows consists of a DISK_SEGMENT structure followed in the case of SEGMENT_PACKET_FORMAT_1
// by the extended fields area (extendedFieldsSize bytes). The extended fields area is a concatenation of
// all the extended fields (including the empty ones). Each extended field is represented as a 1-byte byte
// count (0-255) followed by the actual data bytes.

} segment;

### 3.8 Disk_Segment and ID Structure

The disk segment and related structures which store the global metadata associated with a piece of media.

#### 3.81 Disk_Segment

struct DISK_SEGMENT
{

int prev;D1   // back and forth links: for better symmetry in the routines
int next;D2   // the in use segments on the disk are linked in a ring whose
// "initial" point is the ROOT_SEGMENT, at index=0

int videoClusters;D3  //number of video clusters associated with the segment
int audioClusters;D4  //number of audio clusters associated with the segment

ID id;D5   // The VR ID structure - See separate definition below for ID Structure

// playback procamp values
CHAR videoGain; D28
CHAR videoSetup; D29
CHAR chromaGain; D30// previously resolution LSB
UCHAR hueLSB; D31// previously resolution MSB
UCHAR reserved0;
UINT hueMSB : 2; D32// previously videoContent(=-2); a value <2
// indicates that the procamps are valid
UINT audioTracks:4;D33
UINT writeProtected : 1 ;D34
UINT allocated : 1 ; D35// belongs to the segment list
UINT sliding : 1 ;D36
UINT tcTranslate : 1 ;D37
UINT invisible : 1 ;D38
UINT macro : 1 ;D39
UINT alpha : 1 ;D40
UINT project : 1 ;D41
UINT purged : 1 ;D42
UINT reference : 1 ;D43
UINT looping : 1 ;D44
UINT notReadyToPlay : 1 ;D45
UINT notReadyToTransfer : 1 ;D46
UINT notReadyToArchive : 1 ;D47
UINT transferInProgress : 1 ;D48
UINT reserved:11;D49

};

class DiskSegment
{
public:

};

#### 3.82  ID Structure referred to in DISK_SEGMENT

typedef struct _ID
{

UCHAR code[ID_CODE_SIZE]; D5// 8 format determined by controller
ULONG minFrame; D6// lower limit for the material, used in
// conjunction with start to fully define the
// 1st playable frame
int start; D7// 4 beginning <new> time code ( inclusive )
ULONG duration; D8// 4 length of recorded material
int tcOffset; D9
VIDEO_PARAMS video; D10 //see below for Video Params
int base; D17// base segment of the alias group
int prev;D18 // prev segment in the alias group
int next; D19// next segment in the alias group
VRDATE recordDate; D20// 2 date id was recorded on vr - see below for 2 byte structure of the date field
VRDATE killDate;D21 // 2 date id expires
UCHAR tc_type;D22 // 1 timecode type
UCHAR status; D23// 1 status of id
UCHAR disk; D24// 1 disk ( index to Disk array )
char description[ID_DESCRIPTION_SIZE+1]; D25// 26 description
char agency[ID_AGENCY_SIZE+1]; D26 // 16 material source agency 
char type[ID_TYPE_SIZE+1]; D27// 6 commercial/PSA/etc.

} ID;

#### 3.83 Video Parameters referred to in ID Structure

struct VIDEO_PARAMS
{

ULONG format:4;
ULONG N:7; // GOP size
ULONG M:3; // reference picture period
ULONG bitRate:8;
ULONG vbiPresent:1;
ULONG aspectRatio:1;
ULONG reserved:8;

};

#### 3.84 VR Date Structure referred to in ID Structure

typedef struct _VRDATE
{

USHORT year:7;
USHORT month:4;
USHORT day:5;

} VRDATE;

## 4.0 Sample Application and Code

The attached Xcode.zip file and information provides sample code and instructions that will allow developers to obtain information about how to parse our native stream format from an LXF file. With the included classes and structures, a developer should be able to simply step through the code in debug mode to see what's going on.

The included program is called Transcode.exe, a Harris internal program created to convert a VR native stream format with MPEG2 video into elementary MPEG2 files. It is not intended to run against DV video files. In addition, the transcode application also separates out the audio into a 16-bit, 44.1 Khz WAV file and creates a text file that contains the clip metadata from the Harris file system. (The audio file generation in transcode is known to be unreliable, but is still included to provide a basic understanding of the process.)

NEEDS UPDATE

The source stream is assumed to be in a regular LXF file extracted from a Nexio server. The work involved in obtaining such an LXF file from the Nexio system is not covered here. However, such files can be generated using the “Generic File Read” command, which is covered later in this document.

The function that implements the actual parsing of the VR native stream format is

int CTranscodeDlg::ReadFromFile()

from the TranscodeDlg.cpp file.

Most if not all of the information that defines the native stream format is in PACKET_HEADER.h. In the application, a file is opened and x-amount of bytes are read and passed to a Packet_Header class using a loop:

bytesRead=fread(&ph, sizeof(BYTE), sizeof(ph), sourceFile);

After that, it’s simply a matter of parsing the LXF file to retrieve whatever is required.

## 5.0 Annexes

### 5.1 New LXF Format

As seen in a Hex Editor, this is what the new LXF format looks like, as seen in the Nexio XS, AMP and Preditor product lines.

| Byte Count | Bytes | Hex | Notes |
| --- | --- | --- | --- |
|  |  | Packet Header |  |
| 00 - 07 | 8 | 4C 45 49 54 43 48 00 00 | 1"LEITCH" in ASCII, padded with two 00's |
| 08 - 11 | 4 | 01 00 00 00 | 2Version of this structure: 1=advanced (the old LXF format = 0) |
| 12 - 15 | 4 | 48 00 00 00 | 3Size of header: 72 Dec (48 Hex) |
| 16 - 19 | 4 | ?? 00 00 00 | 4Packet type: video=0, audio=1, or segment=2 |
| 20 - 23 | 4 | 00 00 00 00 | 5Unique stream identifier: currently always 00 |
| 24 - 31 | 8 | Variable | 6Start field number: 8-byte _int64 units represented in 720 kHz units (NTSC frame = 24024, PAL frame = 28800, Audio = Number of samples * 15) |
| 32 - 39 | 8 | Variable | 7Duration in _int64 units represented in 720 kHz units: Segment Packets display the entire clip length, Video Packets are always one frame's worth of material (24024 NTSC 29.97, 28800 PAL 25, 120124 NTSC 59.94, 14400 PAL 50), and Audio Packets will typically be 10 frames of material for NTSC and 2 frames of material for PAL. |
|  |  | Video Packet Header |  |
| 40 -43 | 4 | Variable | 4 bytes representing 32 bits of value (see below). Take Byte values, enter in Little Endian order in calculator, convert to Binary, read in Little Endian order last setting first. Example: 12 C8 03 00, entered in calculator as 03 C8 12, produces 00000000 00000011 11001000 00010010, which gets split up like 00000000 00 00001111 001 0000001 0010 |
|  | 4b | From 0000 – 1111 | V1Video format: (see table 5.1.1, below) |
|  | 7b | From 0000001 – 0001111 | V2N Value of GOP |
|  | 3b | From 001 – 011 | V3M Value of GOP |
|  | 8b | From 00000100 - 00110010 | V4Video Bit Rate (see table 5.1.2, below) |
|  | 2b | From 00 – 11 | V5Picture Type of current frame: I-frame (closed) = 0, I-frame (open) = 1, P-frame = 2, B-frame = 3 |
|  | 8b | 00000000 | V6Reserved |
| 44 – 47 | 4 | Variable | V7Bytes of video data to follow header |
| 48 - 51 | 4 | 00 00 00 00 | V800's |
| 52 – 55 | 4 | Variable | V9Bytes of VBI data to follow header, preceding video data |
| 56 – 59 | 4 | 00 00 00 00 | V1000's |
| 60 – 63 | 4 | 00 00 00 00 | V11Bytes of metadata to follow header, preceding video & VBI data |
| 64 – 67 | 4 | Variable | Checksum values |
| 68 – 71 | 4 | 00 00 00 00 | 4 nulls to provide 8-byte alignment |
|  |  | Audio Packet Header |  |
| 40 - 43 | 4 |  | 4 bytes representing 32 bits of value (see below). Take Byte values, enter in Little Endian order in calculator, convert to Binary, read in Little Endian order last setting first. |
|  | 6b | 010000, 010100, 011000, or 100000 | A3Sample size, in bits: 16, 20, 24, or 32 |
|  | 6b | Variable | A4Sample precision, in bits, in the case that the audio samples are aligned to other variables; for example, 20 bits stuffed into 3 bytes. |
|  | 20b | 0 | A5Reserved |
| 44 - 47 | 4 | Variable | A6Track mask: a bitmap indicating which of 32 possible tracks are included after this header. (Tracks are included from low to high.) For example, 0F=1111= audio channels 1-4 enabled |
| 48 - 51 | 4 | Variable | A7Track size: in bytes, the stride between the starts of two consecutive audio tracks in the data area. This number may exceed the active space taken up by the samples due to alignment considerations. Total data that follows this header should equal track size * number of bits set to one in track mask. |
| 52 - 63 | 12 | 00's | 00's |
| 64 - 67 | 4 | Variable | Checksum values |
| 68 - 71 | 4 | 00 00 00 00 | 4 nulls to provide 8-byte alignment |
|  |  | Segment Packet Header |  |
| 40 - 43 | 4 | 01 00 00 00 | S1Segment Packet Format = 1 always |
| 44 - 47 | 4 | 78 00 00 00 | S2Data size = 120 in Dec |
| 48 - 51 | 4 | Variable | S3Extended fields size: current bytes of data stored |
| 52 - 63 | 12 | 00's | 00's |
| 64 - 67 | 4 | Variable | Checksum values |
| 68 - 71 | 4 | 00 00 00 00 | Null padding to fill out header to 8-byte alignment |
|  |  | Disk Segment |  |
| 72 - 75 | 4 | Variable | D1Previous of the Back & Forth links |
| 76 - 79 | 4 | Variable | D2Next of the Back & Forth links |
| 80 - 83 | 4 | Variable | D3Video clusters - usually small values |
| 84 - 87 | 4 | Variable | D4Audio clusters - usually smaller values (1 or 2) |
| 88 - 95 | 8 | Variable | D58-byte ID |
| 96 - 99 | 4 | Variable | D6Min Frame: lower limit for the material, used in conjunction with Start to fully define the first playable frame |
| 100 - 103 | 4 | Variable | D7Start: Start: beginning time code, set the 0x40 drop frame bit if time is in drop frame NTSC |
| 104 - 107 | 4 | Variable | D8Duration: stated in frames (double for progressive) |
| 108 - 111 | 4 | Variable? | D9Timecode Offset |
| 112-115 | 4 | variable 0 1 00011110 001 0000001 0011 | D104 bytes representing 32 bits of value (see below). Take Byte values, enter in Little Endian order in calculator, convert to Binary, read in Little Endian order last setting first. Example: 12 08 43 00, entered in calculator as 43 08 12, produces 00000000 01000011 00001000 00010010, which gets split up like 00000000 0 1 00001100 001 0000001 0010 |
|  | 4b | From 0000 – 1111 | D11Video format: (see table 5.1.1, below) |
|  | 7b | From 0000001 - 0010000 | D12N Value of GOP |
|  | 3b | From 001 – 011 | D13M Value of GOP |
|  | 8b | From 00000100 - 00110010 | D14Video Bit Rate (see table 5.1.2, below) |
|  | 1b | 0 – 1 | D15VBI present |
|  | 1b | 0 – 1 | D16Aspect Ratio 0-4:3 1-16:9 – Valid for SD content, All HD is assumed 16:9 |
|  | 8b | 00000000 | Reserved |
| 116-119 | 4 | Variable | D17Base segment of the alias group |
| 120-123 | 4 | Variable | D18Previous segment of the alias group |
| 124-127 | 4 | Variable | D19Next segment of the alias group |
| 128-129 | 2 | Variable | D20Record date stored in 2-byte structure (see table 5.1.3, below) |
| 130-131 | 2 | Variable | D21Kill date stored in 2-byte structure (see table 5.1.3, below) |
| 132 | 1 | Variable | D22 Timecode type - 0 = NTSC Nondrop, 1 = NTSC drop, 2 = PAL |
| 133 | 1 | ?? | D23Status |
| 134 | 1 | ?? | D24Disk |
| 135-160 | 26 | Variable | D25Description (old 1): 25-byte field plus 1 (null) |
| 161-176 | 16 | Variable | D26Agency: (old 1) 15-byte field plus 1 (null) |
| 177-182 | 6 | Variable | D27Type: 5-byte field plus 1 (null) |
| 183 | 1 |  | D28Video Gain |
| 184 | 1 |  | D29Video Setup |
| 185 | 1 |  | D30Chroma Gain |
| 186 | 1 |  | D31Hue LSB |
| 187 | 1 | 00 | Reserved |
| 188-191 | 4 | Variable | 4 bytes representing 32 bits of value (see below). Take Byte values, enter in Little Endian order in calculator, convert to Binary, read in Little Endian order last setting first. Example: 92 00 00 00, entered in calculator as 00 00 00 92, produces 00000000 00000000 00000000 10010100, which gets split up like 00000000000 0 0 0 0 0 0 0 0 0 0 0 0 0 1 0 0100 00 |
|  | 2b | 00 – 11 | D32Hue MSB (previously videoContent(=-2); a value <2 indicates that the procamps are valid) |
|  | 4b | 0000 – 1000 | D33Number of Audio Tracks (0-8) |
|  | 1b | 0 or 1 | D342Write Protected bit (Delete Protected) |
|  | 1b | 0 or 1 | D35Allocated bit (belongs to the segment list) |
|  | 1b | 0 or 1 | D36Sliding bit (Looping record) |
|  | 1b | 0 or 1 | D37tcTranslate bit |
|  | 1b | 0 or 1 | D38Invisible bit (Hidden) |
|  | 1b | 0 or 1 | D39Macro bit (Media ID or Consolidated clip) |
|  | 1b | 0 or 1 | D40Alpha bit (Still, roll or crawl) |
|  | 1b | 0 or 1 | D41Project bit (NewsFlash file) |
|  | 1b | 0 or 1 | D42Purged bit (Consolidated clip) |
|  | 1b | 0 or 1 | D43Reference bit |
|  | 1b | 0 or 1 | D44Looping bit |
|  | 1b | 0 or 1 | D45Not Ready To Play bit |
|  | 1b | 0 or 1 | D46Not Ready To Transfer bit |
|  | 1b | 0 or 1 | D47Not Ready To Archive bit |
|  | 1b | 0 or 1 | D48Transfer In Progress bit |
|  | 11b | 000 00000000 | D49Reserved |
|  |  | Extended Fields |  |
| 192 | 1 | 08 |  |
| 193 - 200 | 8 | Variable | Record time-date stamp |
| 201 | 1 | Variable | Size of next field |
| 202 - |  |  | Next field |

1 – The Old Description and Old Agency fields are only available in systems configured for 8- and 32-byte ID systems and when the system does not have the extended fields feature enabled.

Video Format Field: This field provides information about both the video format and the aspect ratio of the indicated clip.

Table 5.1.1: Video Format Table

| Value | Video Format |
| --- | --- |
| 0x00 | Not supported in VR or any NEXIO systems – obsolete JPEG |
| 0x01 | Not supported in VR or any NEXIO systems – obsolete MPEG1 |
| 0x02 | MPEG2 4:2:0 in NEXIO systems, MPEG1 in Preditor systems |
| 0x03 | MPEG2 4:2:2 |
| 0x04 | DVCAM version of DV25 |
| 0x05 | DVCPRO version of DV25 |
| 0x06 | DV50 or DVCPRO HD 1 |
| 0x07 | Uncompressed KRGB 8 bits |
| 0x08 | Uncompressed K 16 bits |
| 0x09 | IMX (MPEG 4:2:2), CBG (constrained bytes per GOP) |
| 0x0A | H.264 4:2:0 |
| 0x0B | H.264 4:2:2 |
| 0x0C | H.264 4:2:0 CBG (e.g. AVC-Intra class 50) |
| 0x0D | H.264 4:2:2 CBG (e.g. AVC-Intra class 100) |
| 0x0E | Reserved 2 |
| 0x0F | Audio-only (no video) |

1	DVCPRO HD Clips are identified by

GetDataRate() == 100 &&

GetVideoFormat() == VIDEO_FORMAT_DVCPRO50

pixelaspect == PIXEL_ASPECT_4_3

2	As of this writing, clips with any of the above H.264 formats are not supported for play out on the NEXIO server platforms. They are only supported for storage and retrieval by Velocity editors.

Video Bit Rate Field: Beginning with NEXIO XS servers and the possibility of recording HD clips, the bit rate field began to support video bit rates above 50 Mb/s. In order to support all the possible values of media within a single byte of data, video bit rate values above 50 are expressed in a progressive slope, as seen in the following table.

Table 5.1.2: Video Bit Rate Table

| Byte Value | Calculation |
| --- | --- |
| ≤ 50 | Bit rate = Value |
| > 50 ≤ 60 | Bit rate = (5 * Value) – 200 |
| > 60 | Bit rate = (10 * Value) – 500 |

Date Fields: 	The data in the Record Date and Kill Date fields include 16-bits which break down as follows:

Table 2-41: Date Format Bitmap

| BIT | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FIELD | Day |  |  |  |  | Month |  |  |  | Year |  |  |  |  |  |  |

The maximum value of these date fields is the last day of the year 2027.

### 5.2 Old LXF Format

As seen in a Hex Editor, this is what the LXF format looks like as used by the Nexio system operating LLM versions 5.07.x, 6.02.x, and 6.04.x and before.

| Byte Count | Bytes | Hex | Notes |
| --- | --- | --- | --- |
|  |  | Packet Header |  |
| 00 - 07 | 8 | 4C 45 49 54 43 48 00 00 | 1"LEITCH" in ASCII, padded with two 00's |
| 08 - 11 | 4 | 00 00 00 00 | 2Version of this structure: 0=current (the new LXF format = 1) |
| 12 - 15 | 4 | 3C 00 00 00 | 3Size of header: 60 Dec (3C Hex) |
| 16 - 19 | 4 | ?? 00 00 00 | 4Packet type: video=0, audio=1, or segment=2 |
| 20 - 23 | 4 | 00 00 00 00 | 5Unique stream identifier: currently always 00 |
| 24 - 27 | 4 | Variable | 6Start field number: 4-byte Integer indicated in fields |
| 28 - 31 | 4 | Variable | 7 Duration in fields: Segment Packets display the entire clip length, Video Packets are always 2, and Audio Packets will typically be 10 for NTSC and 2 for PAL. |
|  |  | Video Packet Header |  |
| 32 - 35 | 4 | variable | 4 bytes representing 32 bits of value (see below). Take Byte values, enter in Little Endian order in calculator, convert to Binary, read in Little Endian order last setting first. Example: 12 C8 03 00, entered in calculator as 03 C8 12, produces 00000000 00000011 11001000 00010010, which gets split up like 00000000 00 00001111 001 0000001 0010 |
|  | 4b | From 0000 - 1001 | V1Video format: JPEG = 0, MPEG1 = 1, Mpeg2 [4:2:0] = 2, Mpeg2 [4:2:2] = 3, Dv25 [4:1:1] or [4:2:0] = 4, DVCPRO = 5, DVCPRO50 = 6, uncompressed KRGB 8 bits = 7, uncompressed K 16 bits = 8, Mpeg2 [4:2:2] Constrained Bytes per GOP = 9 |
|  | 7b | From 0000001 - 0001111 | V2N Value of GOP |
|  | 3b | From 001 - 011 | V3M Value of GOP |
|  | 8b | From 00000100 - 00110010 | V4Video Bit Rate |
|  | 2b | From 00 - 11 | V5Picture Type of current frame: I-frame (closed) = 0, I-frame (open) = 1, P-frame = 2, B-frame = 3 |
|  | 8b | 00000000 | V6Reserved |
| 36 - 39 | 4 | Variable | V7Bytes of video data to follow header |
| 40 - 43 | 4 | 00 00 00 00 | V800's |
| 44 - 47 | 4 | Variable | V9Bytes of VBI data to follow header, preceding video data |
| 48 - 51 | 4 | 00 00 00 00 | V1000's |
| 52 - 55 | 4 | 00 00 00 00 | V11Bytes of metadata to follow header, preceding video & VBI data |
| 56 - 59 | 4 | Variable | Checksum values |
|  |  | Audio Packet Header |  |
| 32 - 35 | 4 | Typically 00 | A1First Active Field; this is needed if an allocation unit has been truncated on a non-5 frame multiple. |
| 36 - 39 | 4 | Typically 0A for NTSC | A2Total fields in packet |
| 40 - 43 | 4 |  | 4 bytes representing 32 bits of value (see below). Take Byte values, enter in Little Endian order in calculator, convert to Binary, read in Little Endian order last setting first. |
|  | 6b | 010000, 010100, 011000, or 100000 | A3Sample size, in bits: 16, 20, 24, or 32 |
|  | 6b | Variable | A4Sample precision, in bits, in the case that the audio samples are aligned to other variables; for example, 20 bits stuffed into 3 bytes. |
|  | 20b | 0 | A5Reserved |
| 44 - 47 | 4 | Variable | A6Track mask: a bitmap indicating which of 32 possible tracks are included after this header. (Tracks are included from low to high.) For example, 0F=1111= audio channels 1-4 enabled |
| 48 - 51 | 4 | Variable | A7Track size: in bytes, the stride between the starts of two consecutive audio tracks in the data area. This number may exceed the active space taken up by the samples due to alignment considerations. Total data that follows this header should equal track size * number of bits set to one in track mask. |
| 52 - 55 | 4 | 00's | 00's |
| 56 - 59 | 4 | Variable | Checksum values |
|  |  | Segment Packet Header |  |
| 32 - 35 | 4 | 01 00 00 00 | S1Segment Packet Format: 0 or 1. 1 is the new way in Nexio for use with extended fields (whether in use or not). 0 is outdated and would only be set for an old unsupported VR system. |
| 36 - 39 | 4 | 78 00 00 00 | S2Data size = 120 in Dec |
| 40 - 43 | 4 | Variable | S3Extended fields size: current bytes of data stored |
| 44 - 55 | 12 | 00's | 00's |
| 56 - 59 | 4 | Variable | Checksum values |
|  |  | Disk Segment |  |
| 60 - 63 | 4 | Variable | D1Previous of the Back & Forth links |
| 64 - 67 | 4 | Variable | D2Next of the Back & Forth links |
| 68 - 71 | 4 | Variable | D3Video clusters - usually small values |
| 72 - 75 | 4 | Variable | D4Audio clusters - usually smaller values (1 or 2) |
| 76 - 83 | 8 | Variable | D58-byte ID |
| 84 - 87 | 4 | Variable | D6Min Frame: lower limit for the material, used in conjunction with Start to fully define the first playable frame |
| 88 - 91 | 4 | Variable | D7Start: beginning time code, set the 0x40 drop frame bit if time is in drop frame NTSC |
| 92- 95 | 4 | Variable | D8Duration: stated in frames |
| 96 - 99 | 4 | Variable | D9Timecode Offset |
| 100-103 | 4 | variable 0 1 00011110 001 0000001 0011 | D104 bytes representing 32 bits of value (see below). Take Byte values, enter in Little Endian order in calculator, convert to Binary, read in Little Endian order last setting first. Example: 12 08 43 00, entered in calculator as 43 08 12, produces 00000000 01000011 00001000 00010010, which gets split up like 000000000 1 00001100 001 0000001 0010 |
|  | 4b | From 0000 - 1111 | D11Video format: JPEG = 0, MPEG1 = 1, Mpeg2 [4:2:0] = 2, Mpeg2 [4:2:2] = 3, Dv25 [4:1:1] or [4:2:0] = 4, DVCPRO = 5, DVCPRO50 = 6, uncompressed KRGB 8 bits = 7, uncompressed K 16 bits = 8, Mpeg2 [4:2:2] Constrained Bytes per GOP (IMX) = 9, None (Audio only) = 15 |
|  | 7b | From 0000001 - 0010000 | D12N Value of GOP |
|  | 3b | From 001 - 011 | D13M Value of GOP |
|  | 8b | From 00000100 - 00110010 | D14Video Bit Rate |
|  | 1b | From 0 - 1 | D15VBI present |
|  | 9b | 000000000 | D16Reserved |
| 104-107 | 4 | Variable | D17Base segment of the alias group |
| 108- 111 | 4 | Variable | D18Previous segment of the alias group |
| 112- 115 | 4 | Variable | D19Next segment of the alias group |
| 116 - 117 | 2 | Variable | D20Record date stored in 2-byte structure |
| 118 - 119 | 2 | Variable | D21Kill date stored in 2-byte structure |
| 120 | 1 | Variable | D22Timecode type - 0 = NTSC Nondrop, 1 = NTSC drop, 2 = PAL |
| 121 | 1 | ?? | D23Status |
| 122 | 1 | ?? | D24Disk |
| 123-148 | 26 | Variable | D25Description: 25-byte field plus 1 |
| 149-164 | 16 | Variable | D26Agency: 15-byte field plus 1 |
| 165 - 170 | 6 | Variable | D27Type: 5-byte field plus 1 |
| 171 | 1 |  | D28Video Gain |
| 172 | 1 |  | D29Video Setup |
| 173 | 1 |  | D30Chroma Gain |
| 174 | 1 |  | D31Hue LSB |
| 175 | 1 | 00 | Reserved |
| 176-179 | 4 | variable | 4 bytes representing 32 bits of value (see below). Take Byte values, enter in Little Endian order in calculator, convert to Binary, read in Little Endian order last setting first. Example: 92 00 00 00, entered in calculator as 00 00 00 92, produces 00000000 00000000 00000000 10010010, which gets split up like 00000000000 0 0 0 0 0 0 0 0 0 0 0 0 0 1 0 0100 10 |
|  | 2b | 00 - 11 | D32Hue MSB (previously videoContent(=-2); a value <2 indicates that the procamps are valid) |
|  | 4b | 0000 - 1000 | D33Number of Audio Tracks (0-8) |
|  | 1b | 0 or 1 | D342Write Protected bit (Delete Protected) |
|  | 1b | 0 or 1 | D35Allocated bit (belongs to the segment list) |
|  | 1b | 0 or 1 | D36Sliding bit (Looping record) |
|  | 1b | 0 or 1 | D37tcTranslate bit |
|  | 1b | 0 or 1 | D38Invisible bit (Hidden) |
|  | 1b | 0 or 1 | D39Macro bit (Media ID or Consolidated clip) |
|  | 1b | 0 or 1 | D40Alpha bit (Still, roll or crawl) |
|  | 1b | 0 or 1 | D41Project bit (NewsFlash or Velocity file) |
|  | 1b | 0 or 1 | D42Purged bit (Consolidated clip) |
|  | 1b | 0 or 1 | D43Reference bit |
|  | 1b | 0 or 1 | D44Looping bit |
|  | 1b | 0 or 1 | D45Not Ready To Play bit |
|  | 1b | 0 or 1 | D46Not Ready To Transfer bit |
|  | 1b | 0 or 1 | D47Not Ready To Archive bit |
|  | 1b | 0 or 1 | D48Transfer In Progress bit |
|  | 11b | 000 00000000 | D49Reserved |
|  |  | Extended Fields |  |
| 180 | 1 | 08 |  |
| 181 - 188 | 8 | Variable | Record time-date stamp |
| 189 | 1 | Variable | Size of next field |
| 190 - |  |  | Next field |

### 5.3 Extended Fields

Listed below in a very specific order are the extended fields that are part of the extended metadata we store within the LLM.

To properly make use of these extended fields, you must meet the following requirements:

- The LLM raid set must be resized for 64-byte IDs

- The LLM raid set must be resized for an extended fields setting of 1024

- Each field has a pre-determined size as listed below

- At the very end of the list below is a reserved section. This is for critical fields we will need in the future that we could not anticipate today. Do not store anything in this field!

Items in red are data fields stored in Unicode

| Field # | MAX Size | Chars | Field Name | Purpose | Generator |
| --- | --- | --- | --- | --- | --- |
| 0 | 8 | n/a | Record Date/Time | The time and date stamp when a clip was recorded. This provides better information than the existing Record Date field in the original MediaBase field list | LLM |
| 1 | 19 | 19 | Codec Where Recorded | The VR computer name and VR Codec where the clip was recorded, in the format of "VR440-YX/VRn" | LLM |
| 2 | 8 | 8 | Source 8-byte Handle | Info stored in a low-res system to indicate the 8-byte ID Handle of the matching hi-res clips | LLM |
| 3 | 64 | 64 | UMID | Unique identifier | LLM |
| 4 | 16 | 128 bits | HD Video size / rate info | Extra info about the video for ASI, MXF. (see below) | LLM |
| 5 | 4 | 32 bits | Source Video Info | Info stored in a low-res system to indicate the video format information about a source clip in the high-res system | Encoder |
| 6 | 16 | 128 bits | GUID | Globally Unique IDentifier | LLM |
| 7 | 64 | 32 | User Name | Log in name, typically associated with clips that the identified user initiated records on | NXOS, NF |
| 8 | 64 | 32 | Department | Clip categorization to group clips by | NXOS, NF |
| 9 | 64 | 32 or 64 | Reserved | Reserved | Reserved |
| 10 | 12 | 12 | Reserved | Reserved | Reserved |
| 11 | 64 | 32 | Link | ClipSync field to link two clips together | User |
| 12 | 240 | 120 | Extended Desc. | Move this field to fast access data field | User |
| 13 | 30 | 15 | Extended Agency | Move this field from original field which is too small in Unicode | User |
| 14 | 50 | 25 | User-definable Field #1 | Text entry field | User |
| 15 | 50 | 25 | User-definable Field #2 | Text entry field | User |
| 16 | 50 | 25 | User-definable Field #3 | Text entry field | User |
| 17 | 50 | 25 | User-definable Field #4 | Text entry field | User |
| 18 | 16 | 16 | External Controller UID | Unique identifier for Third Party Controllers | Third Party App |
| 19 | 14 | 14 | Video ARC | Video Aspect Ratio Conversion | User |
| 20 | 8 | 8 | Modified Timestamp | Time clip was last modified in MBP | LLM |
| 21 | 2 | 2 | Video QA Status | Video quality analysis (see below) | VideoTek |
| 22 | 2 | 2 | User Segments In Use (bitmask) | Used for managing the information stored in user data segments, part of the DTA modes. The 2 bytes represent a bitmask of “in use” user data slots | LLM |
| 23 | 32 | 32 | Audio Track Info. | A series of 2-byte data per audio track (up to 16 tracks) to indicate each track’s grouping with other tracks, audio codec, and language (see detail below) |  |
| 24 | 53 | Reserved | For future use and definition | Reserved |  |

5.31 Extended Field 4 (Video Info)
This is a 128-bit bitmap represented LSB first (bitmap position-0 is byte-0, bit-0) that records ALL Mpeg formats encountered throughout the spot and complements the old style video params, which indicate just the "highest" value encountered. Note that the ext field 4 bitmap is not filled before some valid mpeg data arrives during open-ended WriteFile's (CF A1), for non-Mpeg spots, or if not enough ext fields space is available.

// description of the video info bitmap (128 bits), stored as EXTENDED_FIELD_VIDEO_INFO (ext field 4)
#define VIDEO_INFO_FORMATS (VERTICAL_SIZE_CODES*SCAN_TYPES*FRAME_RATE_CODES) // 96

enum VIDEO_INFO_BIT
{
// Format bits, bit # = formatCode, where
//
// formatCode= (verticalsizeCode * SCAN_TYPES + scanType) * FRAME_RATE_CODES + frameRateCode;
//
// reverse formulae:
//
// frameRateCode=formatCode%FRAME_RATE_CODES;
// temp=formatCode/FRAME_RATE_CODES;
// scanType=temp%SCAN_TYPES;
// verticalsizeCode=temp/SCAN_TYPES;
//
VIDEO_INFO_FORMAT_BASE=0, // 0..95

// Picture structure bits:

// if (Mpeg) pictureStructure!=PICTURE_STRUCTURE_FRAME
VIDEO_INFO_PICTURE_FIELD=VIDEO_INFO_FORMAT_BASE+VIDEO_INFO_FORMATS, // 96
// else if (Mpeg) progressiveFrame==0
VIDEO_INFO_PICTURE_FRAME_INTERLACED, // 97
// else if (Mpeg) repeatFirstField==0
VIDEO_INFO_PICTURE_FRAME_PROGRESSIVE, // 98
// else (Mpeg pull-down frame)
VIDEO_INFO_PICTURE_FRAME_PROGRESSIVE_REPEAT, // 99

// Chroma format bits (see description below):

// = if (Mpeg 4:2:0 or invalid) chroma_format<=CHROMA_FORMAT_420 
VIDEO_INFO_CHROMA_420, // 100
// else (Mpeg 4:2:2 or 4:4:4)
VIDEO_INFO_CHROMA_422, // 101
VIDEO_INFO_CHROMA_311, // 102

// HD Pixel aspect bits (see description below):

VIDEO_INFO_PIXEL_ASPECT_1_1, // 103 HD with pixel aspect = 1:1
VIDEO_INFO_PIXEL_ASPECT_4_3, // 104 HD with pixel aspect = 4:3

VIDEO_INFO_BITS=128
};

where:

enum VERTICAL_SIZE_CODE // unofficial
{
VERTICAL_SIZE_CODE_480=0,
VERTICAL_SIZE_CODE_512,
VERTICAL_SIZE_CODE_576,
VERTICAL_SIZE_CODE_608,
VERTICAL_SIZE_CODE_720,
VERTICAL_SIZE_CODE_1080,
VERTICAL_SIZE_CODES
};

enum SCAN_TYPE // = (Mpeg) progressive_sequence
{
SCAN_TYPE_INTERLACED=0,
SCAN_TYPE_PROGRESSIVE=1,
SCAN_TYPES
};

enum FRAME_RATE_CODE // unofficial = (Mpeg) frame_rate_code - 1
{
FRAME_RATE_CODE_24M=0,
FRAME_RATE_CODE_24,
FRAME_RATE_CODE_25,
FRAME_RATE_CODE_30M,
FRAME_RATE_CODE_30,
FRAME_RATE_CODE_50,
FRAME_RATE_CODE_60M,
FRAME_RATE_CODE_60,
FRAME_RATE_CODES
};

use of VIDEO_INFO_PIXEL_ASPECT_x_y bits (*):
---------------------------------------------------------------------------
VIDEO_INFO_PIXEL_ASPECT_1_1 = "pixel w/h = 1" set for the following HD bitmap sizes: 1920x1080, 1280x720
VIDEO_INFO_PIXEL_ASPECT_4_3 = "pixel w/h = 4/3" set for the following HD bitmap sizes: 1440x1080, 960x720
(*) for backward compatibility, if neither bit is set then all HD material in the spot shall be assumed of VIDEO_PIXEL_ASPECT_1_1

use of videoFormat, VIDEO_INFO_CHROMA_xxx bits, and bitRate:
-------------------------------------------------------------------------------------------------------
VIDEO_FORMAT_MPEG2_MpMl for encoding = MPEG, chroma = 4:2:0
VIDEO_FORMAT_MPEG2_MpPl for encoding = MPEG, chroma = 4:2:2
VIDEO_FORMAT_DVCPRO for encoding = HDCAM proprietary, chroma = 3:1:1, in conjunction with bitRate = BitRate32To8(140)
VIDEO_FORMAT_DVCPRO50 for encoding = DVCPRO-HD, chroma = 4:2:2, in conjunction with bitRate = BitRate32To8(100)

To help decode the format code value retrieved from the Video Info field, use the following reverse formula:

frameRateCode 	= formatCode % FRAME_RATE_CODES;

temp 	= formatCode / FRAME_RATE_CODES;

scanType 	= temp % SCAN_TYPES;

verticalsizeCode	= temp / SCAN_TYPES;

5.32 Extended Field 21 (Video QA Status)


This is a 16-bit bitmap representing the quality of a particular video clip based on analysis tools from Videotek.

Byte 1 Content:

0x00 = "None" -- Initial state on ingest
0x01 = "Passed"
0x02 = "Failed"
0x03 = "Analyzing"
0x04 = "Pending"

Byte 2 Content:

0x00 -- Initial state on ingest - reserved for future use

5.32 Extended Field 22 (Index of  Active User Segments)

The bitmask corresponds to the following table of user data segments.

| Segment Number | Hex Value | Description |
| --- | --- | --- |
| 16 | 0x10 | Legacy Extended Description data - no longer part of Nexio Systems that support Unicode |
| 17 | 0x11 | Locator information for the NewsFlash |
| 18 | 0x12 | Settings for the VR465 NewsFlash - settings windows and lists |
| 19 | 0x13 | Settings for the VR475 NewsFlash - settings windows and lists |
| 20 | 0x14 | NewsFlash user settings |
| 21 | 0x15 | NXOS user settings |
| 22 | 0x16 | Velocity user settings |
| 23 | 0x17 | FCP user settings |
| 24 | 0x18 | Locator information for the Velocity |
| ... |  |  |
| 30 | 0x1E | Virtual Path and Long File Name for CIFS |
| 31 | 0x1F | Videotek video quality inspection information |

VR and NEXIO Note: Field 22 is not supported on the VR or the original NEXIO servers. It is specific to the NEXIO AMP and XS servers only.

5.33 Extended Field 23 (Audio Track Info):

The size of this field is 32 bytes. The structure of the Audio Track Info field is one byte per audio track up to a maximum of 32 audio tracks. The one byte is broken down as follows:

Table 5.31.1: Audio Track Info

| Byte/Bits | Description |
| --- | --- |
| Bits 0 – 3 | Indicates associated group, used to lock channels together in the interface If PCM AUDIO(b4-7=0) it used to identify track relationships (e.g. stereo components) 0 = ungrouped >0 = grouped with If COMPRESSED AUDIO(b4-7>0) b0 0 = single track 1 = AES pair b1-3 = number of decoded tracks |
| Bits 4 – 7 | Indicates the audio format of the track where: 0 = PCM 1 = Neural 2 = Dolby AC3 3 = Dolby-E 4 = MPEG Layer 1 and 2 |

VR and NEXIO Note: Field 23 is not supported on the VR or the original NEXIO servers. It is specific to the NEXIO AMP and XS servers only.

5.34 Extended Field 24 (Audio Tag Info):

The Audio Tag Info field is a way to organize information about the different tracks associated with a clip. It is used primarily with the Track Editor tool which can manipulate audio tracks so that different languages associated with the clip play out of different channels on the NEXIO server.

The size of this field is 32 bytes. The structure of the Audio Track Info field is one byte per audio track up to a maximum of 32 audio tracks. The one byte indicates the track's audio language code based on the ISO 639 Windows definition: http://unicode.org/onlinedat/languages.html.

VR and NEXIO Note: Field 24 is not supported on the VR or the original NEXIO servers. It is specific to the NEXIO AMP and XS servers only.

5.34 Example of Extended Field Decoding

LXF EXAMPLE – Decoding the start of the file (as seen by hex editor)

Offset													Data

0x00	4C 45 49 54 43 48 00 00 01 00 00 00 48 00 00 00 02 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00

0x20	00 2E 93 02 00 00 00 00 01 00 00 00 78 00 00 00 8E 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00

0x40	1F 43 23 A9 00 00 00 00 D5 04 00 00 65 04 00 00 D3 00 00 00 06 00 00 00 25 30 30 30 30 34 36 32

0x60	00 00 00 00 00 00 00 00 DC 05 00 00 00 00 00 00 16 08 CF 00 62 04 00 00 62 04 00 00 62 04 00 00

0x80	6C 1B EC 1B 02 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00

0xA0	00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 B4 00 90 00 00 00

0xC0	08 40 BD D6 D2 FF C5 C8 01 10 4E 58 33 36 30 31 48 44 58 2D 31 2F 43 48 33 00 00 00 10 00 00 00

0xE0	00 00 00 00 00 00 20 00 00 A4 00 00 00 00 10 FF 4C 7A 77 5D 74 2C 44 92 63 BB 79 AE 5A FF 20 1A

0x100	41 00 64 00 6D 00 69 00 6E 00 69 00 73 00 74 00 72 00 61 00 74 00 6F 00 72 00 00 00 00 00 00 0E

0x120	37 00 32 00 30 00 70 00 2D 00 35 00 30 00 00 00 00 00 00 0E 00 00 00 00 00 00 00 00 00 00 00 00

0x140	00 00 08 B0 B9 E6 88 D6 FD C8 01 02 00 00 4C 45 49 54 43 48 00 00 01 00 00 00 48 00 00 00 00 00

0x160	00 00 00 00 00 00 00 00 00 00 00 00 00 00 40 38 00 00 00 00 00 00 16 08 0F 00 00 65 04 00 00 00

0x180	00 00 88 16 00 00 00 00 00 00 00 00 00 00 4A B6 A2 AB 00 00 00 00 06 00 0D 8B 1A 7E 1B 84 10 7D

0x1A0	13 86 16 7C 18 88 14 7D 18 88 19 7D 15 87 17 7C 17 83 17 7C 17 7F 16 7F 13 82 15 7F 15 86 12 7C

0x1C0	13 85 15 7B 15 84 13 7E 10 82 12 7F 16 7E 17 7D 15 7E 15 7A 23 8A 2F 79 2E 97 2C 75 2B 9C 25 72

0x1E0	28 9E 33 73 32 A1 30 71 42 A3 59 67 56 A4 4B 61 50 A6 4B 65 40 A8 3D 6A 38 A8 2E 70 26 AA 2F 72

PACKET HEADER, SEGMENT PACKET HEADER, DISK SEGMENT, EXTENDED FIELDS, VIDEO PACKET HEADER, VIDEO DATA

Extended field bloc from the example, bytes 0x30–0x33 in the Segment Packet Header set the extended field length at 0x8E

Decoding the extended fields from byte 0xC0:

OffSet													Data

0xC0	08 40 BD D6 D2 FF C5 C8 01 10 4E 58 33 36 30 31 48 44 58 2D 31 2F 43 48 33 00 00 00 10 00 00 00

0xE0	00 00 00 00 00 00 20 00 00 A4 00 00 00 00 10 FF 4C 7A 77 5D 74 2C 44 92 63 BB 79 AE 5A FF 20 1A

0x100	41 00 64 00 6D 00 69 00 6E 00 69 00 73 00 74 00 72 00 61 00 74 00 6F 00 72 00 00 00 00 00 00 0E

0x120	37 00 32 00 30 00 70 00 2D 00 35 00 30 00 00 00 00 00 00 0E 00 00 00 00 00 00 00 00 00 00 00 00

0x140	00 00 08 B0 B9 E6 88 D6 FD C8 01 02 00 00 4C 45 49 54 43 48 00 00 01 00 00 00 48 00 00 00 00 00

08	EXTENDED FIELD TAG LEN

00 	UNUSED EXTENDED FIELD TAG

45	START OF NEXT PACKET HEADER

Extended Field 	Length  Value

0      		08     	40 BD D6 D2 FF C5 C8 01

1      		10     	4E 58 33 36 30 31 48 44 58 2D 31 2F 43 48 33 00

2			00	 	UNUSED

3			00     	UNUSED

4			10		00 00 00 00 00 00 00 00 00 20 00 00 A4 00 00 00

5      		00     	UNUSED

6			10		FF 4C 7A 77 5D 74 2C 44 92 63 BB 79 AE 5A FF 20

7			1A		41 00 64 00 6D 00 69 00 6E 00 69 00 73 00 74 00 72 00 61 00 74 00 6F 00 72 00

8			00		UNUSED

9      		00     	UNUSED

10     		00     	UNUSED

11     		00     	UNUSED

12     		00     	UNUSED

13     		0E     	37 00 32 00 30 00 70 00 2D 00 35 00 30 00

14     		00     	UNUSED

15     		00     	UNUSED

16     		00     	UNUSED

17     		00     	UNUSED

18     		00     	UNUSED

19     		0E     	00 00 00 00 00 00 00 00 00 00 00 00 00 00

20     		08     	B0 B9 E6 88 D6 FD C8 01

21     		02     	00 00